4X6 HDMI Switch/Splitter Extender over CAT.5e/6

HD4600T

User’s Manual

Version 1.0

1. Introduction

1.

Thanks for purchasing the 4 x 6 HDMI Switch/Splitter Extender over CAT.5e/6.

Please read this

Manual and retain it for reference in future if needed.

1.1 Features

Supports 1 source selected among 4 HDMI sources and the same source content can be done splitting displaying on remote all of 6 displays and even distributing with extending through Ethernet switch 10/100.

LED indicators monitor for connection status

Supports HDMI 1.3, HDCP compliant

Supports resolution up to Full HD 1080p, 1920 x 1080 @60Hz

Each corresponding extender receiver unit (remote) links cascade-chainable additional 2 receivers for expansion.

Extended distance over CAT. 5e/6 up to 100m (point-to-point)

HDMI sources selected by front buttons

IR (Infrared Remote)

Enabled for communication of between the correspondence of

switch (Emitter cable) and receiver unit (RX) – (Receiver cable)

RS232 enabled for each communication of between the correspondence of switch and receiver unit (RX)

Supports local loop HDMI monitoring and Audio enabled

1.2 Package Contents

2.

3.

4 x 6 HDMI Switch/Splitter Extender (TX) x 1

DC 5V / 2A Power Adapter x 1

Audio Cable 1.8m x 1

1

4.  RS-232 Cable x 4

5.  IR Emitter Cable x 4

6.  Rack Mount Bracket x 2 (1 sets)

7.  User’s Manual x 1

2. Specifications

Console

4 x 6 HDMI Switch/Splitter Extender (Transmitter)

Connectors

RJ-45

HDMI source

Extension Port

Audio

IR

Power

LED Indicators

Link

Selected HDMI

source

HDMI Source Select Button

DDC Supported

Extension Cable Type & Length

Max. Video Resolution Supported

Housing material

Operating Temperature

Power Supply

Dimension (L x W x H)

Humidity

6 (Line Out)

HDMI Output

1 x HDMI (Female)

RS-232Control Port   4 x 3.5Ø jack

HDMI Input

RJ-45

4 x HDMI (Female)

Full HD 1080p Extension

Supports Stereo Audio

Unidirectional 38KHz

Red LED

Green LED

Green LED for selected HDMI source

5

DDC, DDC2B

CAT.5e / CAT.6 Max. Length: 100m (point-to-point)

Full HD 1080p, 1920 x 1080 @60Hz

Metal

32 - 122 F (0 - 50 C)

External DC 5V / 2A Power Adapter

275 x 98 x 38 mm

0% - 80% RH

2

3. Connection between Switch Extender (Transmitter) and Receiver

Transmitter

Receiver

Choose one available RJ-45 port of the Extender Receiver (RX) connected with the

Switch Extender (Transmitter) over CAT.5e or CAT.6 cable.

Remarks:

The Network Switch 10/100 enough could be transceiver for extending distance.

Please refer to the following connections:

(2). The Network Switch is possibly connected between Receiver and Receiver.

(1). The Network Switch is linked between Switch Extender (Transmitter) and Receiver.

5

4.  IR Connection

5.  RS-232 Connection

Connect the RS-232 adapter cable to Switch Extender (Transmitter) RS-232 3.5mm jack, and another

end of the cable is connected to the RS-232 port of PC.

Connect the RS-232 adapter cable to Extender Receiver (RX)’s RS-232 3.5mm jack, and another end

of the cable is connected to the RS-232 port of the 3rd party’s device.

6

Disclaimer

Information in this document is subject to change without notice. The manufacturer does not make any representations or warranties

(implied or otherwise) regarding the accuracy and completeness of this document and shall in no event be liable for any loss of profit

or any other commercial damage, including but not limited to special, incidental, consequential, or other damages.

No part of this document may be reproduced or transmitted in any form by any means, electronic or mechanical, including

photocopying, recording or information recording and retrieval systems without the express written permission of the manufacturer.

All brand names and product names used in this document are trademarks, or registered trademarks of their respective holders.

FCC Statement

This device generates and uses radio frequency and may cause interference to radio and television reception if not installed and

used properly. This has been tested and found to comply with the limits of a Class B computing device in accordance with the

specifications in Part 15 of the FCC Rules. These specifications are designed to provide reasonable protection against such

interference in a residential installation. However, there is no guarantee that interference will not occur in a particular installation. If

this device does cause harmful interference to radio or television reception, which can be determined by plugging the device in and

out, the user can try to correct the interference by one or more of the following measures:

Reorient or relocate the receiving antenna.

Increase the separation between the device and receiver.

Connect the computer into an outlet on a circuit different from that to which the receiver is connected.

Consult the dealer or an experienced radio/TV technician for help.

CE / FCC

2R40M4088